#### Converter MPC-210 (Code 702.042)

## Technical passport

Kaunas

© 2013

# Device purpose

- Data exchange between RS232/RS485/I2C and Ethernet;

# Device delivery set

- Converter MPC-210	1pc.

- Technical passport	1 pc.

- Software	1 pc.

# Technical data

##### Overall specification

CPU: 	ARM Cortex M3

Memory: archive storage 1-8 MB, battery backup, valid up to 5 years.

##### Interfaces

RS232:	up to 15m, speed up to 19,2 Kbits/s

RS485:	up to 1.2 km, max 32 transceivers, speed up to 19.2 Kbits/s

Ethernet:	10/100 Mb twisted pair, up to 100m

USB:	mini-B USB ver. 2.0

##### LED indication

Power

Serial port read/write for each port

Ethernet status

##### Protocols

Modbus RTU, Modbus TCP/IP, IP, ICMP, UDP, TCP, DHCP, PPP, ARP, SNTP

##### General

Power supply AC:	50-250V ~ 50/60Hz

Power supply DC:	50-400V

Power over Ethernet: 	PoE (IEEE 802.3af) - see manufacturer's code

Power consumption :	<300mA

Interface galvanic isolation:	1000V

Real time clock:		Yes

##### Physical characteristics

Dimensions:	18x85x97 mm

Weight:	105 g

Case fixing type:	DIN35

Protection type:	IP20

##### Climate conditions

Operating temperature:	-25 .. +60oC

Storage temperature:	-40 .. +60oC

Humidity range:	5 – 95%, non- condensing

##### Programing and updating

Remotely: 	Ethernet

Locally: 	USB

# Model code

# Model code explanation

##### 702.042.A.B

A- First socket (UART1)

| 1 | RS485 |
| --- | --- |
| 2 | RS232 |
| 3 | I2C |

B- Power

| 1 | 50-250VAC/50-400VDC |
| --- | --- |
| 2 | PoE (Power over Ethernet) |

# Manufacturer warranty

Warranty period – 24 months.

In case of controller’s unavailability, the manufacturer commits to replace or carry out repair free of charges, if device was used in accordance with the operating manual.

A guarantee does not apply to faults caused by force majeure.

# Putting into operation certificate

##### Controller MPC-210

Model code: 702.042.2.1

Serial number: 85971

S.P.

Date of manufacturing:  February   2016

Controller is correctly assembled and completed.

The person responsible for certification: Rūta Mockutė